#include <Keyboard.h>

const int leftButtonPin = A1;   // Pin for the left button (A1)

const int rightButtonPin = A2;  // Pin for the right button (A2)

const int groundPin = A0;       // Pin for ground (A0)

void setup() {

pinMode(leftButtonPin, INPUT_PULLUP);   // Initialize the left button with internal pull-up resistor

pinMode(rightButtonPin, INPUT_PULLUP);  // Initialize the right button with internal pull-up resistor

pinMode(groundPin, OUTPUT);             // Initialize the ground pin as output

digitalWrite(groundPin, LOW);           // Set ground pin to low

Keyboard.begin();                      // Initialize keyboard emulation

}

void loop() {

// Check button presses

checkButtonPresses(leftButtonPin, 'l');

checkButtonPresses(rightButtonPin, 'r');

}

void checkButtonPresses(int buttonPin, char key) {

static bool buttonState = HIGH;    // Button state (HIGH - not pressed, LOW - pressed)

static bool lastButtonState = HIGH;    // Previous button state

bool currentState = digitalRead(buttonPin);  // Current button state

// If button state has changed

if (currentState != lastButtonState) {

// If button is pressed

if (currentState == LOW) {

pressKey(key);  // Emulate key press

}

}

lastButtonState = currentState;  // Update previous button state

}

void pressKey(char key) {

Keyboard.press(key);  // Press the key

delay(1000);  // Delay after key press for 1 second

Keyboard.release(key);  // Release the key

}